# Attachment 7. Compliance List - Specifications for Infrastructure& e-Portal 2/2

# 1. Overall

# 1. NIC

# 1) NIC Overall

| No | Classification | Specification | Unit | Q'ty | Reference in proposal where this is covered |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | English version | Spanish version |
|  | e-Portal Hardware | H/W and S/W | Set | 1 |  |  |
| 1 | Innovation Center Operation Office | H/W and S/W | Set | 1 |  |  |
|  |  | Interior | Set | 1 |  |  |

# 2) e-Portal Hardware

| No | Equipment | Specification | Unit | Q'ty | Compliance (Y/N) Partially compliant | Bidder’s Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Web Server for users and administrator | - 2U Rack Type - 4 Core Intel Xeon 2.4GHz/10MB L3 Cache * 2EA - Memory: 16GB - HDD: 300GB 10K SAS * 2EA, RAID 1 - HBA: 8Gb FC * 2EA - LAN: 1Gbps Fiber Ethernet Card * 2EA - ODD: DVD RW - Dual Power Supply - OS: Linux | Set | 3 |  |  |
| 2 | WAS Server | - 2U Rack Type - 4 Core Intel Xeon 2.4GHz/10MB L3 Cache * 2EA - Memory: 16GB - HDD: 300GB 10K SAS * 2EA, RAID 1 - HBA: 8Gb FC * 2EA - LAN: 1Gbps Fiber Ethernet Card * 2EA - ODD: DVD RW - Dual Power Supply - OS: Linux | Set | 2 |  |  |
| 3 | LMS Server | - 2U Rack Type - 4 Core Intel Xeon 2.4GHz/10MB L3 Cache * 2EA - Memory: 8GB - HDD: 300GB 10K SAS * 2EA, RAID 1 - HBA: 8Gb FC * 2EA - LAN: 1Gbps Fiber Ethernet Card * 2EA - ODD: DVD RW - Dual Power Supply - OS: Linux | Set | 1 |  |  |
| 4 | DB Server | - 4U Rack Type - 8 Core Intel Xeon 2.13GHz/24MB L3 Cache * 4EA - Memory: 32GB - HDD: 300GB 10K SAS * 2EA, RAID 1 - HBA: 8Gb FC * 2EA - LAN: 1Gbps Fiber Ethernet Card * 2EA - ODD: DVD RW - Dual Power Supply - OS: Window Server 2008 R2 Enterprise | Set | 2 |  |  |
| 5 | LDAP Server | - 2U Rack Type - 4 Core Intel Xeon 2.4GHz/10MB L3 Cache * 2EA - Memory: 8GB - HDD: 300GB 10K SAS * 2EA, RAID 1 - HBA: 8Gb FC * 2EA - LAN: 1Gbps Fiber Ethernet Card * 2EA - ODD: DVD RW - Dual Power Supply - OS: Linux | Set | 2 |  |  |
| 6 | Content and Uploading Server | - 2U Rack Type - 4 Core Intel Xeon 2.4GHz/10MB L3 Cache * 2EA - Memory: 8GB - HDD: 300GB 10K SAS * 2EA, RAID 1 - HBA: 8Gb FC * 2EA - LAN: 1Gbps Fiber Ethernet Card * 2EA - ODD: DVD RW - Dual Power Supply - OS: Linux | Set | 1 |  |  |
| 7 | Search Engine Server | - 2U Rack Type - 4 Core Intel Xeon 2.4GHz/10MB L3 Cache * 2EA - Memory: 8GB - HDD: 300GB 10K SAS * 2EA, RAID 1 - HBA: 8Gb FC * 2EA - LAN: 1Gbps Fiber Ethernet Card * 2EA - ODD: DVD RW - Dual Power Supply - OS: Linux | Set | 1 |  |  |
| 8 | CMS Server | - 2U Rack Type - 4 Core Intel Xeon 2.4GHz/10MB L3 Cache * 2EA - Memory: 8GB - HDD: 300GB 10K SAS * 2EA, RAID 1 - HBA: 8Gb FC * 2EA - LAN: 1Gbps Fiber Ethernet Card * 2EA - ODD: DVD RW - Dual Power Supply - OS: Linux | Set | 1 |  |  |
| 9 | Media Server with VoD Transcoding | - 2U Rack Type - 4 Core Intel Xeon 2.4GHz/10MB L3 Cache * 2EA - Memory: 16GB - HDD: 300GB 10K SAS * 2EA, RAID 1 - HBA: 8Gb FC * 2EA - LAN: 1Gbps Fiber Ethernet Card * 2EA - ODD: DVD RW - Dual Power Supply - OS: Linux | Set | 1 |  |  |
| 10 | SMS Server | - 2U Rack Type - 4 Core Intel Xeon 2.4GHz/10MB L3 Cache * 2EA - Memory: 8GB - HDD: 300GB 10K SAS * 2EA, RAID 1 - HBA: 8Gb FC * 2EA - LAN: 1Gbps Fiber Ethernet Card * 2EA - ODD: DVD RW - Dual Power Supply - OS: Linux | Set | 1 |  |  |
| 11 | DBMS Server for Digital Repository System | - 2U Rack Type - 4 Core Intel Xeon 2.4GHz/10MB L3 Cache * 2EA - Memory: 8GB - HDD: 300GB 10K SAS * 2EA, RAID 1 - HBA: 8Gb FC * 2EA - LAN: 1Gbps Fiber Ethernet Card * 2EA - ODD: DVD RW - Dual Power Supply - OS: Linux | Set | 1 |  |  |
| 12 | Web Analytic Server & Reporting Server | - 2U Rack Type - 4 Core Intel Xeon 2.4GHz/10MB L3 Cache * 2EA - Memory: 8GB - HDD: 300GB 10K SAS * 2EA, RAID 1 - HBA: 8Gb FC * 2EA - LAN: 1Gbps Fiber Ethernet Card * 2EA - ODD: DVD RW - Dual Power Supply - OS: Linux | Set | 2 |  |  |
| 13 | Backup Server | - 2U Rack Type - 4 Core Intel Xeon 2.4GHz/10MB L3 Cache * 2EA - Memory: 8GB - HDD: 300GB 10K SAS * 2EA, RAID 1 - HBA: 8Gb FC * 2EA - LAN: 1Gbps Fiber Ethernet Card * 2EA - ODD: DVD RW - Dual Power Supply - OS: Linux | Set | 1 |  |  |
| 14 | Mass Mail Server | - Processor: Intel core i5-2400 processor (3.10 GHZ,6MB) and above - Graphics: 1GB or above - Memory: 6GB(DDR3 1333MHZ/4GB+2GB) - HDD: 1TB(7200RPM S-ATA2) - Monitor: 22" LCD Monitor - O/S: Windows 7 Home Premium(64-bit) - Keyboard: Spanish and English - Mouse: USB Type - Including Anti-Virus Program | Set | 1 |  |  |
| 15 | Backup Device | - Backup methods LTO 5 - Drives: 4 EA - Slot: 48 or more - Support capacity per: 1.5/3TB(uncompressed / compressed) - Built-in Barcode Reader - A variety of OS environments (Solaris, HP-UX, AIX, etc.) can be configured Including Cartridge 50Pack, FC Cable | Set | 1 |  |  |
| 16 | SAN Switch | - Ports: 24 EA or more (Including SFP) - Type of Port : Fiber Channel Interface(E_Port, F_Port, FL_Port, M_Port) - Link speed: 8,4,2.1 Gbps auto-sensing - Dual power supply & fan | Set | 2 |  |  |
| 17 | SAN Storage | - Dual Active / Active configuration. - Controller liver failure, fail-over support - Offers more than 16GB of data cache - Host Interface: 8Gbps FC 8 port, 1Gbps iSCSI 4 port - Capacity: usable 20TB (900GB 2.5 in. 10k SAS HDD) - RAID 0, 1, 5, 6, 10 support - The major components (Power, Fan, Controller, etc.) redundancy | Set | 1 |  |  |
| 18 | NAS Storage | - Dual Active / Active configuration. - Controller liver failure, fail-over support - Controller NAS: Per controller supports - Controller FC: Per controller supports - Offers more than 16GB of data cache - NAS: Provides more than 72GB of data cache - Host Interface: 8Gbps FC 8 port, 1Gbps iSCSI 4 port - NAS interface 1Gbps 4 port, 10Gbps 4 ports provide more than - Capacity : usable 24TB (900GB 2.5 in. 10k SAS HDD) - RAID 0, 1, 5, 6, 10 support - Storage virtualization can support - The major components (Power, Fan, Controller, etc.) redundancy | Set | 1 |  |  |
| 19 | Monitoring PC | - Processor: Intel core i5-2400 processor (3.10 GHZ,6MB) and above - Graphics: 1GB or above - Memory: 6GB(DDR3 1333MHZ/4GB+2GB) - HDD: 1TB(7200RPM S-ATA2) - Monitor: 22" LCD Monitor - O/S: Windows 7 Home Premium(64-bit) - Keyboard: Spanish and English - Mouse: USB Type Including Anti-Virus Program | Set | 1 |  |  |
| 20 | L4 Switch | - Layer 4~7 Switching - Backplane: 16Gbps or more - Concurrent Session: 4,000,000 or more - 1G Fiber Ethernet: 8ports or more (Including SFP) | Set | 2 |  |  |
| 21 | Backbone Switch | - 9slots chassis - Backplane: 720Gbps or more - Dual Switch Fabric Engine - 10G Ethernet: 16ports or more - 1G Fiber Ethernet: 24ports or more (Including SFP Cable) - 10/100/1000 Mbps Ethernet: 48ports or more - Dual Power Supply | Set | 2 |  |  |
| 22 | Workgroup Switch | - Backplane: 32Gbps or more - 1G Fiber Ethernet: 4ports or more (Including SFP) - 10/100/1000 Mbps Ethernet: 24ports or more | Set | 2 |  |  |
| 23 | RACK | - 42U Rack, PDU - Only one(1) rack for KVM Switch, Panel Monitor | Set | 5 |  |  |
| 24 | Cable & Accessory for installation | - Including cable & accessory for equipment installation | Set | 1 |  |  |

# 3) NIC Operation Office

- H/W and S/W

| No | Equipment | Specification | Unit | Q'ty | Compliance (Y/N) Partially compliant | Bidder’s Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Operator Computer | - Processor: Intel core i5-2400 processor (3.10 GHZ,6MB) and above - Graphics: 1GB and above - Memory: 6GB(DDR3 1333MHZ/4GB+2GB) - HDD: 1TB(7200RPM S-ATA2) - Monitor: 22" LCD Monitor - O/S: Windows 7 Home Premium (64-bit) - Keyboard: Spanish and English - Mouse: USB Type Including Anti-Virus Program | Set | 13 |  |  |
| 2 | Digital Copier | - Color Print, Copy, Scan, Fax Printing - Print Speed (Black): Up to 25 ppm - Print Speed (Color): Up to 25 ppm - Print Resolution (dpi): Up to 600 x 600 dpi Copy - Copy Speed (Black): Up to 25 cpm - Copy Speed (Color): Up to 25 cpm - Copy Resolution: 600 x 600 dpi Scan - Method: Color CCD or CIS or Flatbed - Scan Speed: 20 ipm - Resolution (Optical): 600 x 600 dpi - Resolution (Enhanced): 4,800 x 4,800 dpi Fax - Compatibility: ITU-T G3 General - Interface: USB 2.0, Ethernet 10/100/1000 Base TX - Memory: 1GB Including ADF | Set | 1 |  |  |
| 3 | Multi Function Color Printer | Color Print, Copy, Scan, Fax Printing - Print Speed (Black): 21ppm - Print Speed (Color): 21ppm - Paper Input Capacity (Standard): 250 sheets Copy - Copy Speed (Black): 21cpm - Copy Speed (Color): 21cpm  Scan - Method: Color CCD or CIS or Flatbed - Scan Speed(Black): 11ipm or higher Fax - Modem Speed (kbps): 33.6 - Compatibility: ITU-T G3 - Fax Memory: 4MB or up to 250pages General - Interface: USB 2.0, Ethernet 10/100 Base TX - Memory: 256MB | Set | 1 |  |  |
| 4 | UPS for PC | - Output Capacity: 450Watts / 750 VA - Max Configurable Power: 450 Watts / 750 VA - Input /Output Voltage: 120V, 50/60Hz ± 3Hz - Battery Type: sealed Lead-Acid battery - Typical recharge time: 16hour - Backup Time at Half Load: 11minute(225 Watts) Comply with electricity standards of Colombia | Set | 13 |  |  |
| 5 | Video Projector & Screen | Projector - Native Resolution: XGA (1024x768) - Brightness (ANSI-Lumens): 4000ANSI - Contrast Ratio (FIFO): 2000:1 and above - Lamp life (hours): 2500 and above Portable Screen - Size: 120”(2400 x 1800) - Material: Matt White | Set | 1 |  |  |
| 6 | DAS/NAS Storage for RIC content backup | - 2.0GHz CPU, 512MB DDR2 Memory, 16MB Flash memory - HDD: 8TB(4 X 2TB, 3.5" SATA) - Tray: 4 X Hot Swappable Tray - Network: 2 Gigabit Ethernet - USB: 4 X USB 2.0 - Form Factor: Tower | Set | 1 |  |  |
| 7 | Consumables & Stationery | - Toner for Multifunction Color Printer (Black, Minimum Printing Capacity : 2,000pages): 5EA - Toner for Multifunction Color Printer (Yellow, Minimum Printing Capacity : 2,000pages): 5EA - Toner for Multifunction Color Printer (Magenta, Minimum Printing Capacity : 2,000pages): 5EA - Toner for Multifunction Color Printer (Cyan, Minimum Printing Capacity : 2,000pages): 5EA - Toner for Digital Copier (Black, Minimum Printing Capacity : 2,000pages): 3EA - Toner for Digital Copier (Yellow, Minimum Printing Capacity : 2,000pages): 3EA - Toner for Digital Copier (Magenta, Minimum Printing Capacity : 2,000pages): 3EA - Toner for Digital Copier (Cyan, Minimum Printing Capacity : 2,000pages): 3EA | Set | 1 |  |  |
| 8 | Software | MS-Office , Home and Business or above(latest) | Set | 13 |  |  |
| 9 | Workgroup Switch | - Backplane: 32Gbps or more - 1G Fiber Ethernet: 4ports or more (Including SFP) - 10/100/1000 Mbps Ethernet: 24ports or more | Set | 1 |  |  |
| 10 | Power & Network Cabling | - Network Cabling(minimum 15 port): Cat 7A and miscellaneous materials - Power Cabling: HIV 600 heat-resistant Cable and power plugs (Colombian standard Type) - Comply with electricity/communication standards/regulations | Set | 1 |  |  |

- Interior

| No | Equipment | Specification | Unit | Q'ty | Compliance (Y/N) Partially compliant | Bidder’s Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Furniture | - Office Desk: 13EA - Partition: 1Set - Chair: 21EA - Printer Desk: 4EA - Office Cabinet: 13EA - Conference table(8 person): 1EA - File Cabinet: 13EA * Reference only and minimum requirements | Set | 1 |  |  |
| 2 | Interior | - Wall, Roof, Window, etc * The Bidder shall propose a space plan. | Set | 1 |  |  |

# 2. RIC

# 1) RIC Overall

| No | Classification | Specification | Unit | Q'ty | Reference in proposal where this is covered |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | English version | Spanish version |
| 1 | Server and Storage System | Hard ware & Soft ware | Set | 5 |  |  |
| 2 | RIC Operation Office | H/W and S/W | Set | 5 |  |  |
|  |  | Interior | Set | 5 |  |  |
| 3 | Multimedia Studio | H/W and S/W | Set | 5 |  |  |
|  |  | Studio Soundproof Construction | Set | 5 |  |  |
| 4 | Content Development and Research Office | H/W and S/W | Set | 5 |  |  |
|  |  | Interior | Set | 5 |  |  |
| 5 | Supporting Office (Central RIC) | H/W and S/W | Set | 1 |  |  |
|  |  | Interior | Set | 1 |  |  |
| 6 | Supporting Office (Other 4 RIC) | H/W and S/W | Set | 4 |  |  |
|  |  | Interior | Set | 4 |  |  |
| 7 | Multimedia Classroom Type A | H/W and S/W | Set | 5 |  |  |
|  |  | Interior | Set | 5 |  |  |
| 8 | Multimedia Classroom Type B | H/W and S/W | Set | 5 |  |  |
|  |  | Interior | Set | 5 |  |  |
| 9 | General Classroom | H/W and S/W | Set | 10 |  |  |
|  |  | Interior | Set | 10 |  |  |
| 10 | RIC Security System | RIC Security System | Set | 5 |  |  |

# 2) Server and Storage system

- H/W and S/W

| No | Equipment | Specification | Unit | Q'ty | Compliance (Y/N) Partially compliant | Bidder’s Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Test Server | - 2U Rack Type - 4 Core Intel Xeon 2.4GHz/10MB L3 Cache * 2EA - Memory: 8GB - HDD: 300GB 10K SAS * 2EA, RAID 1 - LAN: 1Gbps Fiber Ethernet Card * 2EA - ODD: DVD RW - Dual Power Supply - OS: Linux | Set | 1 |  |  |
| 2 | Digital Resource Management Server | - 2U Rack Type - 4 Core Intel Xeon 2.4GHz/10MB L3 Cache * 2EA - Memory: 8GB - HDD: 300GB 10K SAS * 2EA, RAID 1 - LAN: 1Gbps Fiber Ethernet Card * 2EA - ODD: DVD RW - Dual Power Supply - OS: Linux | Set | 1 |  |  |
| 3 | DAS/NAS Storage | - Processor: Intel Xeon Quad-Core Processor X3480 processor 3.06GHz+ - Memory: 8192 MB DDR3 SDRAM - HDD Bays: 16 * 3.5"/2.5" SATA III or SAS 6Gb/s Disk - RAID Level: 0, 1, 5,6,10, 50,60 및 JBOD - Interface: SATA/SAS (x16) via backplane - Power: "Redundant power supply least 700W - NIC: Intel 10GbE X520-DA2 Dual port with SFP GBIC Module - HDD: 24TB (2,000GB 7.2K RPM SATA III Disk * 12EA) - High Availability Support (HA) - Access Guard: allows or denies specific IP or MAC addresses | Set | 1 |  |  |
| 4 | Workgroup Switch | - Backplane: 32Gbps or more - 1G Fiber Ethernet: 4ports or more (Including SFP) - 10/100/1000 Mbps Ethernet: 24ports or more | Set | 7 |  |  |
| 5 | Rack | - 42U Rack & KVM Switch, PDU, Panel Monitor | Set | 1 |  |  |

# 3) RIC Operation Office

- H/W and S/W

| No | Equipment | Specification | Unit | Q'ty | Compliance (Y/N) Partially compliant | Bidder’s Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Operator Computer | - Processor: core i5-2400 processor (3.10 GHZ, 6MB) and above - Graphics: 1GB or above - Memory: 6GB(DDR3 1333MHZ/4GB +2GB) - HDD: 1TB (7200RPM S-ATA2) - Monitor: 22" LCD Monitor - O/S: Windows 7 Home Premium (64-bit) - Keyboard: Spanish and English - Mouse: USB Type - Including Anti-Virus Program | Set | 7 |  |  |
| 2 | Digital Copier | - Color Print, Copy, Scan, Fax Printing - Print Speed (Black): Up to 25 ppm - Print Speed (Color): Up to 25 ppm - Print Resolution (dpi): Up to 600 x 600 dpi Copy - Copy Speed (Black): Up to 25 cpm - Copy Speed (Color): Up to 25 cpm - Copy Resolution: 600 x 600 dpi Scan - Method: Color CCD or CIS or Flatbed - Scan Speed: 20 ipm - Resolution (Optical): 600 x 600 dpi - Resolution (Enhanced): 4,800 x 4,800 dpi Fax - Compatibility: ITU-T G3 General - Interface: USB 2.0, Ethernet 10/100/1000 Base TX - Memory: 1GB Including ADF | Set | 1 |  |  |
| 3 | Multi Function Color Printer | Color Print, Copy, Scan, Fax Printing - Print Speed (Black): 21ppm - Print Speed (Color): 21ppm - Paper Input Capacity (Standard): 250 sheets Copy - Copy Speed (Black): 21cpm - Copy Speed (Color): 21cpm  Scan - Method: Color CCD or CIS or Flatbed - Scan Speed(Black): 11ipm or higher Fax - Modem Speed (kbps): 33.6 - Compatibility: ITU-T G3 - Fax Memory: 4MB or up to 250pages General - Interface: USB 2.0, Ethernet 10/100 Base TX - Memory: 256MB | Set | 1 |  |  |
| 4 | Consumables & Stationery | - Toner for Multifunction Color Printer (Black, Minimum Printing Capacity : 2,000pages): 3EA - Toner for Multifunction Color Printer (Yellow, Minimum Printing Capacity : 2,000pages): 3EA - Toner for Multifunction Color Printer (Magenta, Minimum Printing Capacity : 2,000pages): 3EA - Toner for Multifunction Color Printer (Cyan, Minimum Printing Capacity : 2,000pages): 3EA - Toner for Digital Copier (Black, Minimum Printing Capacity : 2,000pages): 2EA - Toner for Digital Copier (Yellow, Minimum Printing Capacity : 2,000pages): 2EA - Toner for Digital Copier (Magenta, Minimum Printing Capacity : 2,000pages): 2EA Toner for Digital Copier (Cyan, Minimum Printing Capacity : 2,000pages): 2EA | Set | 1 |  |  |
| 5 | UPS for PC | - Output Capacity: 450Watts / 750 VA - Max Configurable Power: 450 Watts / 750 VA - Input /Output Voltage: 120V, 50/60Hz ± 3Hz - Battery Type: sealed Lead-Acid battery - Typical recharge time: 16hour - Backup Time at Half Load: 11minute(225 Watts) - Comply with electricity standards of Colombia | Set | 7 |  |  |
| 6 | Video Projector & Screen | Projector - Native Resolution: XGA (1024x768) - Brightness (ANSI-Lumens): 4000ANSI - Contrast Ratio (FIFO): 2000:1 and above - Lamp life (hours): 2500 and above Portable Screen - Size: 120”(2400 x 1800) - Material: Matt White | Set | 1 |  |  |
| 7 | Software | MS-Office , Home and Business or above(latest) | Set | 7 |  |  |
| 8 | Power & Network Cabling | - Network Cabling(minimum 9 Port): 7A and miscellaneous materials - Power Cabling: HIV 600 heat-resistant Cable and power plugs of Colombian Type - Compliant with Colombian electricity standards | Set | 1 |  |  |

- Interior

| No | Equipment | Specification | Unit | Q'ty | Compliance (Y/N) Partially compliant | Bidder’s Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Furniture | - Office Desk: 7EA - Partition: 1Set - Chair: 15EA - Printer Desk: 1EA - Office Cabinet: 7EA - Conference table(8person) 1EA - File Cabinet: 7EA * Reference only and minimum requirements | Set | 1 |  |  |
| 2 | Interior | - Wall, Roof, Window, etc - Air Conditioner * The Bidder shall propose a space plan. | Set | 1 |  |  |

# 4) Multimedia Studio

- H/W and S/W

| No | Equipment | Specification | Unit | Q'ty | Compliance (Y/N) Partially compliant | Bidder’s Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | HD Camera System | HD Camera system - Sensor: 1/3 Type 3Cmos - Total pixels per sensor: 2.37 Mega pixels - Effective pixels per sensor: 2.07 Mega-pixels - Resolution: 1000 tv lines or more - Zoom ratio: 18X - Recording file format: MXF - Recording format: MPEG 2 LONG GOP - Recording frame rate: - 50Mbps: 1920 x 1080/60i,30p,24p; 1280x720/60p,30p, 24p - 35Mbps: 1920 x 1080/60i,30p,24p; 1280x720/60p,30p, 24p - 25Mbps: 1440 x 1080/60i,30p, 24p - HD / SD-SDI OUTPUT: YES - Time cord, Component Out function Including Battery Pack, Carrying case, Remote Controller, CF Card Reader, Rain Cover, Memory 32GB 2ea | Set | 2 |  |  |
| 2 | CCU for HD Camera System | Camera Control Unit - Focus: Focus dial, Focus direction switch, Focus auto On / Off Key, Expanded focus key - Iris: Iris dial, Iris direction switch, Iris auto On / Off key - Zoom: Zoom lever, Zoom direction switch, Zoom Speed Switch - White Balance: WHT BAL key, Preset key - Gain: Gain key, Select Key - Shutter: Shutter Key - Camera control ch: 5ch | Set | 1 |  |  |
| 3 | W/L Microphone System for HD Camera | Transmitter - Frequency response: 40 Hz ~ 18 KHz - Audio input connector: 3-pole mini jack Receiver - Frequency response: 40 Hz ~ 18 KHz - Signal to noise ratio: 60dB or more *Proposed products shall have frequency range that is acceptable in Colombia. | Set | 1 |  |  |
| 4 | Boom Microphone System | - Frequency Range: 20Hz - 20kHz - Maximum SPL: 131dB SPL - Sensitivity: -36.0dB re 1 Volt/Pascal - Output: XLR Including Boom pole, Wind screen | Set | 1 |  |  |
| 5 | Tripod System | Tripod System for HD Camera with Prompter Head - Payload range: 2 to 25kg / 4 to 55 lbs - Counterbalance: 16 steps, plus Boost Button - Tilt range: +90 ~ -70 - For tripods with 100mm bowls Tripod - Load Capacity: >40㎏ - Weight: <2.6㎏ - Min. Height: 460㎜ - Max. Height: 1,550㎜ - Material: Carbon fiber Spreader - Load Capacity: >50㎏ - Weight: < 0.8㎏ Cover - Weight: < 3.81㎏ - Material: Polyethylene Including Pan Bar, Dolly | Set | 1 |  |  |
|  |  | Tripod System for HD Camera only Head - Counter balance: 5 + 0 - Max. load: >4㎏ - Pan angle range: 360deg - Tilt angle range: > +90˚ to -75˚ - Grade of drag: 3 and 0 - Tripod interface: 75㎜ bowl Tripod - Material: Dural aluminum - Max. Height: >1,500㎜ - Min. Height: <405㎜ - Load Capacity: >20㎏ Spreader - Load capacity: > 40㎏ - Weight: < 0.6㎏ Including Dolly, Carrying Bag | Set | 1 |  |  |
| 6 | Studio LED Line Monitor | - Screen Size: 42" - Resolution: 1920 x 1080p - Display Type: LED - Aspect Ratio: 16:9 Including TV Stand | Set | 1 |  |  |
| 7 | Studio Monitor Speaker | - Type: Bi-amp 2-way powered speaker - Frequency response: 55Hz –20kHz - Output Power : 70W(Dynamic Power) | Set | 2 |  |  |
| 8 | Prompter | - Display size: 20" (508mm) - Brightness: 1000 Nits - Resolution: 1280*1024 - Contrast ratio: 1000:1 - Input signal: BNC(Composite),VGA,SDI - Reading Range: 7m(23ft) Including QStart Software, USB Shuttle Xpress Control | Set | 1 |  |  |
| 9 | Notebook for Prompter | - CPU: Intel Core i5-560M (2.66GHz) - Display: LED backlight / 13.3inch - Panel resolution: 1366x768 - HDD: 320GB - Memory: 2GB DDR3 - OS: Window 7 Professional - Including DVD recorder, 6cell, 1Gbps LAN, 802.11n wireless LAN, HDMI, usb2.0 | Set | 1 |  |  |
| 10 | HD Switcher | Video - Input - 8 simultaneous live video sources, in any combination of HD-SDI, HD Component, SD-SDI, SD Component, Y/C (BNC) or Composite connections and resolutions - Virtual: 8 independent Input - Video Output: Up to 5 output connections and 2 independent A/V output signals - Mix and match output formats, connection types, resolutions and aspect ratios - HD-SDI, HD Component, SD-SDI, SD Component, Y/C (BNC) or Composite - HDMI, DVI and VGA output for displays or projector - Network output for live streaming - Recording: Multi-track, multi-format recording of up to 8 simultaneous channels - Native recording in up to 1080p with time code - Live Streaming: HD live streaming via Gigabit connection Audio - Inputs: - 8 SDI Embedded - 8 AES3/EBU - 8 x 2 Balanced XLR (Microphone/Line) - Phantom Power Support - Outputs: -3 SDI Embedded - 2 AES3/EBU - 4 Balanced XLR - 4 Balanced XLR (AUX) - 1 Stereo 1/4” (phones) - Supported Formats: NTSC: 1080/30p, 1080/24p, 1080/60i, 720/60p, 720/30p, 720/24p, 480/60i Multi-Standard: NTSC-J; PAL 1080/25p, 1080/50i, 720/50p, 720/25p, 576/25i - Playback Media Formats: AVI, DV, DVCPro, DVCProHD, FLV, F4V, H.263, H.264, MOV, MKV, MJPEG, MPEG (1, 2, all profiles, program or trans-port streams), MP4, WMV, WebM, PSD, PNG, TGA, BMP, JPEG, EXR, RAW, TIF, AIFF, MP3, WAV, (Including Apple ProRes) - Processing: - Video: 4:4:4:4, 32-bit Floating Point - Audio: 4 channels, 96 kHz, 32-bit Floating Point - Monitoring: Selectable layouts for user interface and multi-view preview displays | Set | 1 |  |  |
| 11 | Digital Video Deck | - HD Format Connections - SDI Video Input: 4 x 10-bit SD/HD/3 Gb/s SDI switchable - SDI Video Output:4 x 10-bit SD/HD/3G Gb/s SDI switchable - SDI Audio Input:16 channels embedded in SD and HD - SDI Audio Output: 16 channels embedded in SD and HD - Codec Support: Uncompressed QuickTime, Apple ProRes 422 (HQ) QuickTime, Avid DNxHD QuickTime, Avid DNxHD MXF - 2K format support : 2048x1080p25 - Displays: Built-in LCD for video, audio and time code monitoring Including SSD 128GB 2EA | Set | 2 |  |  |
| 12 | 2D/3D Non-Linear Editing System | Editing Software - Premiere Pro CS6 Workstation - MS Windows 7 Professional 64-bit OS - E5-2640 2.50Ghz 15MB 1333MHz 6Core X 1EA - Graphic : 1GB and above - 16GB (4x4GB) DDR3 - 1000GB 1st HDD - 1000GB 2nd HDD - 16X DVD+-RW Super Multi SATA 1st Drive - USB Standard Keyboard - 22 inch monitor 2EA Storage - Host Interface: Dual 4Gb Fiber Channels - Type: Single RAID Controller - CPU: Intel Xscale i80348@800Mhz - RAID level: 0, 1, 10, 3, 5, 6 , 30, 50, 60 , Single Disk and JBOD - Bay: Hotswap 8ea - Capacity : 16TB(2,000GB X 8EA) - HDD: Option(SATAII,SATA,ATA) - Cache: Standard 1GB - Power: Redundant by Dual 460W Power modules - Module - Drive Tray X 8EA - Embedded Web browser-based RAID manager | Set | 1 |  |  |
| 13 | LCD Multi-view Monitor | - Screen Size: 42" - Resolution: 1920 x 1080p - Display Type: LED - Aspect Ratio: 16:9 Including TV Stand | Set | 1 |  |  |
| 14 | PGM Monitor | - Size: 17inch - Resolution: 1366X768 - Contrast ratio: 900:1 - Input: 2 X HD/SD SDI, 1xDVI, Analog Component / Composite / S-video / RGB - Output: 1 X HD/SD SDI through-out | Set | 1 |  |  |
| 15 | HDV VTR | - Recording format: HDV1080/60i, 1080/50i, 1080/24p, 1080/25p, 1080/30p - Playout format: HDV1080/60i, 1080/50i, 1080/24p, 1080/25p, 1080/30p. - Video input/output: BNC x1/BNC x1 - S-video input/output: Mini-DIN 4pin x 2 - Component video output: BNC x 3 - Digital uncompressed output: HDMI Connector x 1 - Audio input: RCA x2, Stereo / Audio output: RCA x2,stereo - LCD monitor: 2.7-inch type | Set | 1 |  |  |
| 16 | Sync Generator | - HD Sync: Tri-level - SD Sync: Color Black, 75% Color Bars - Accuracy: 3 ppm - User Controls: External Dipswitch | Set | 1 |  |  |
| 17 | Blue Ray Player | - HDMI out: 19 pin (Type A, HDMI™ Connector) - Digital Audio Output : Optical | Set | 1 |  |  |
| 18 | HD-SDI Router | - Port: 8 Input / 8 Output - Signal: 10 Bit SD-SDI, HD-SDI, 3G-SDI - Video Rate: 270 Mb/s ~ 3 Gb/s - Connector: BNC Jack, 75 Ohms - Audio: Embedded, TV Standard 48KHz 24Bit Sampling | Set | 1 |  |  |
| 19 | Audio Distribution AMP | Input - Number of Input:1In ( 2 Channels L/R) - Impedance: 600 ohm or 20K ohm balanced  Output - Number of Output: 6 Out ( 2 Channels L/R) - Loop Output: 1 Out (2 Channels L/R) - Impedance: 600 ohm balanced Output | Set | 1 |  |  |
| 20 | Interactive Display Board | - Size: 82", LED Panel - 16:9 aspect ratio - Resolution: 1920×1080(1080P, Full HD - Brightness: 450(cd/㎡) - Contrast : 5000:1 - PC Interface : USB, Plug and Play Including cables and accessories, S/W | Set | 1 |  |  |
| 21 | Audio Mixer | 16ch digital audio mixer - Chanel: 1~16 Microphones/Lines Analog input - CH Input 1 to 12: Microphones & Lines - CH Input 13 to 16: Lines - 2 TR In [L, R]: 600Ω Lines Analog Output - Stereo Out [L,R]: 600Ω Lines - Omni Out(1 to 4): 10KΩ Lines - Monitor Out(L,R): 10KΩ Lines - 2TR out (L,R): 10KΩ Lines Digital Input - 2TR IN Digital: RCA Pin Jack - ADAT In: Optical Digital Output - 2TR Out Digital: RCA Pin Jack - ADAT Out: Optical | Set | 1 |  |  |
| 22 | Control Room Monitor Speaker | - Type: Bi-amp 2-way powered speaker - Frequency response: 55Hz – 20kHz - Output Power : 70W(Dynamic Power) | Set | 2 |  |  |
| 23 | Wired Microphone | - Pick-up pattern: cardioid - Frequency response: 80 – 15,000 Hz - Sensitivity (free field, no load, 1 kHz): 1.5 mV/Pa - Nominal impedance: 350 Ω - Min. terminating impedance: 1 kΩ Including Stand | Set | 2 |  |  |
|  | Gooseneck Microphone | - Polar Pattern: Hyper-cardioid - Frequency response: 70 Hz - 18 kHz - Maximum Sound Level: 124 dB SPL - Connector: XLR-3 male - Equivalent Acoustic Noise: 39 dBA SPL Including Stand | Set | 1 |  |  |
| 24 | W/L Microphone | - Frequency response: 25~18000Hz Receiver - Antenna connector: 2x BNC - Audio output level (balanced): XLR Transmitter - RF output power typ. 30 mW - Power supply 2x AA batteries Microphone - Transducer principle : condenser - AF sensitivity: 20 mV/Pa omni directional - Sound pressure level (SPL): 130 dB(SPL) max. *Comply with frequency rate of Colombia | Set | 2 |  |  |
| 25 | Headphone | - System: Dynamic - Frequency range: 12Hz to 28kHz - Sensitivity: 125dB SPL/V - Input impedance: 32 Ohms - Max. input power: 30mW - Cable: 1m cable | Set | 1 |  |  |
| 26 | Network Switch | - Connectors: 16 10BASE-T/100BASE-X/1000BASE-T, 4 Gigabit SFP ports - Performance: 40 Gbps switching capacity, max. / 29.6 Mpps forwarding rate, max | Set | 1 |  |  |
| 27 | Intercom System | - Panel Microphone Input Type: Electret - Headset Microphone Input Type: Dynamic - Speaker Output - Load Impedance: >= 4Ω - Frequency Response(Panel Mic): 600 - 10KHz ± 3dB Including Headset 3EA, Belt-pack 2EA | Set | 1 |  |  |
| 28 | Back-up System | - LTO Backup system - Sustained Transfer Rate - 100MB/s direct attached, - up to 75 MB/s network - Hard Disk Drive - 2000 GB X 2 – RAID0/1 - Format Compatibility - GNU or POSIX tar v1.20 or later, LTFS 2.0 Direct Attached File Systems Supported - Drive - Native Capacity: 1500 GB - Average Access Time: 52 Sec from BOT - Rewind Time: 96 Sec from EOT - Average Load Time: >19 Seconds - MTBF: 250,000 Hours - Uncorrected Error Rate: 1x1017 - LTO 5 Media - Archival Life — 30 years - Durability — 5,000 load/unload cycles Including LTO5 1.5TB Tape 10ea, Cleaning tape 1ea, Rack mount Kit | Set | 1 |  |  |
| 29 | On Air Lamp | - Lamp Type: Red LED Type - Tally Input: 10mA | Set | 2 |  |  |
| 30 | UPS | - Output Power Capacity: 10 KVA - Input - Voltage Range: 90~130AVC - Frequency: 50/60Hz ± 5% - Max Input Current: 112A - Output - Voltage Range: 110AVC± 1% - Frequency: 50/60Hz ± 5% - AVR Function: included - Battery Type Maintenance-free sealed Lead-Acid battery - Typical recharge time: 90% capacity after 8 hours - Surge Protection - Backup Time: 30 minutes or less Comply with electricity standards of Colombia | Set | 2 |  |  |
| 31 | Chroma key Screen | - 150” Electronic Screen - 3000 X 4000 (mm) - Color: Blue | Set | 1 |  |  |
| 32 | Console Desk | - Material: Wood and Steel - Number of Bay: 5 Bay - Base: Rack Type | Set | 1 |  |  |
| 33 | NLE Desk | - Material: Wood - desk for 2D/3D Editing System | Set | 1 |  |  |
| 34 | Power Distributer | - AC Voltage meter: Line input / output (use of the voltage) - Channel: 8 Ch - Ch load: 10A or more - Total max load: 45A or more - Power: 55 watts or more - Comply with electricity standards of Colombia | Set | 2 |  |  |
| 35 | System Rack | - 19", 42U - Aluminum | Set | 1 |  |  |

- Studio Soundproof Construction

| No | Equipment | Specification | Unit | Q'ty | Compliance (Y/N) Partially compliant | Bidder’s Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Soundproof Interior | - Soundproof system and doors, dustproof system, Furniture - Soundproof Door - Materials : Al 1.5T ~ 2.0T - Sound Reduction Index : 35 ~ 45dB - Soundproof Window - Triplex Soundproof Glasses - Materials : St’l 2.3T, Al 3.0T - Sound Reduction Index : 40 ~ 50dB * The bidder shall submit a certificate of soundproof test issued by an authorized agency in the country of origin. - Air conditioner 2 Set or more - Extinguishing System : Carbon Dioxide Extinguishing System or Halogen Compounds Extinguishing System * Reference only – The Bidder shall propose an optimized space plan. *Comply with electricity standards of Colombia | Set | 1 |  |  |
| 2 | Power Cabling and Installation | - Electric Power Cabling - Audio/Visual Signal Cabling - Equipment Installation * For Audio/Visual Signal Cabling, Cables and connectors from"Beldon" or "Gepco" are advised. * Comply with electricity standards of Colombia | Set | 1 |  |  |
| 3 | Studio Lighting | - Light Fly Duct: 5000L 4EA - Upper Horizon Light: 300W 9EA - Spot Light: 650W, 3EA - Back Spot Light: 2EA - Photo Optic Light(POL 4/220W): 10EA - Barn Door: 3EA - Light Hanger: 24EA - Safety Chain: 24EA - R/D Unit Panel & Control Board(12RD) - Pipe Grid * Reference only – The Bidder shall propose an optimized space plan. *Proposed lightings shall be low-heat emitting products. *Comply with electricity standards of Colombia | Set | 1 |  |  |

# 5) Content Development and Research Office

- H/W and S/W

| No | Equipment | Specification | Unit | Q'ty | Compliance (Y/N) Partially compliant | Bidder’s Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Operator Computer | - Processor: Intel core i5-2400 processor (3.10 GHZ,6MB) and above - Graphics: 1GB or above - Memory: 6GB(DDR3 1333MHZ/4GB +2GB) - HDD: 1TB (7200RPM S-ATA2) - Monitor: 22" LCD Monitor * 2EA - O/S: Windows 7 Home Premium(64-bit) - Keyboard: Spanish and English - Mouse: USB Type Including Anti-Virus Program | Set | 10 |  |  |
| 2 | Multi Function Color Printer | Color Print, Copy, Scan, Fax Printing - Print Speed (Black): 21ppm - Print Speed (Color): 21ppm - Paper Input Capacity (Standard): 250 sheets Copy - Copy Speed (Black): 21cpm - Copy Speed (Color): 21cpm  Scan - Method: Color CCD or CIS or Flatbed - Scan Speed(Black): 11ipm or higher Fax - Modem Speed (kbps): 33.6 - Compatibility: ITU-T G3 - Fax Memory: 4MB or up to 250pages General - Interface: USB 2.0, Ethernet 10/100 Base TX - Memory: 256MB | Set | 2 |  |  |
| 3 | UPS for PC | - Output Capacity: 450Watts / 750 VA - Max Configurable Power: 450 Watts / 750 VA - Input /Output Voltage: 120V, 50/60Hz ± 3Hz - Battery Type: sealed Lead-Acid battery - Typical recharge time: 16hour - Backup Time at Half Load: 11minute(225 Watts) - Comply with electricity standards of Colombia | Set | 10 |  |  |
| 4 | Digital Copier | - Color Print, Copy, Scan, Fax Printing - Print Speed (Black): Up to 25 ppm - Print Speed (Color): Up to 25 ppm - Print Resolution (dpi): Up to 600 x 600 dpi Copy - Copy Speed (Black): Up to 25 cpm - Copy Speed (Color): Up to 25 cpm - Copy Resolution: 600 x 600 dpi Scan - Method: Color CCD or CIS or Flatbed - Scan Speed: 20 ipm - Resolution (Optical): 600 x 600 dpi - Resolution (Enhanced): 4,800 x 4,800 dpi Fax - Compatibility: ITU-T G3 General - Interface: USB 2.0, Ethernet 10/100/1000 Base TX - Memory: 1GB Including ADF | Set | 1 |  |  |
| 5 | Digital Camera | - Effective Pixel: Approx. 20.3 Mega-pixels - Total Pixel: Approx. 21.6 Mega-pixels - Image Sensor Type: CMOS - Main LCD Screen: AMOLED - Resolution: VGA ( 640X480 ) 614k dots - USB 2.0 (HI-SPEED) - Video Out: NTSC, PAL (user selectable), HDMI 1.4a: (1080i, 720P, 576P / 480P) - Storage media: SD / SDHC / SDXC - Including Tripod, Rechargeable Battery, Bag, Lens(18~55mm Lens), 32GB Memory card, Compatible Microphone | Set | 2 |  |  |
| 6 | HD Camcorder | - Type: 1080p HD Switch Grip Camcorder - Lens: Optical Zoom: 20x, Digital Zoom: 40x - No. of Pixels: 5.1M Pixel - Sensor Type: 1/4" (effec.1/6.3") - Display: 2.7" Wide touch - Recording Format: H.264 HP - Resolution: Full HD i (1920x1080/60i), Vertical HD(1920x1080/30p), HD p (1280x720/60p) SD D1 (720x480/60P) - Video File Format: MP4 - Audio Recoding Format: AAC - Storage media: SD / SDHC / SDXC including Tripod, Bag, 32GB Memory card | Set | 2 |  |  |
| 7 | Video Projector & Screen | Projector - Native Resolution: XGA (1024x768) - Brightness (ANSI-Lumens): 4000ANSI - Contrast Ratio (FIFO): 2000:1 and above - Lamp life (hours): 2500 and above Portable Screen - Size: 120”(2400 x 1800) - Material: Matt White | Set | 1 |  |  |
| 8 | Consumables & Stationery | - Toner for Multifunction Color Printer (Black, Minimum Printing Capacity : 2,000pages): 5EA - Toner for Multifunction Color Printer (Yellow, Minimum Printing Capacity : 2,000pages): 5EA - Toner for Multifunction Color Printer (Magenta, Minimum Printing Capacity : 2,000pages): 5EA - Toner for Multifunction Color Printer (Cyan, Minimum Printing Capacity : 2,000pages): 5EA - Toner for Digital Copier (Black, Minimum Printing Capacity : 2,000pages): 3EA - Toner for Digital Copier (Yellow, Minimum Printing Capacity : 2,000pages): 3EA - Toner for Digital Copier (Magenta, Minimum Printing Capacity : 2,000pages): 3EA Toner for Digital Copier (Cyan, Minimum Printing Capacity : 2,000pages): 3EA | Set | 1 |  |  |
| 9 | CD/DVD Duplicator | - 1:11 Duplication - Memory: 128MB - Internal HDD: 500GB - Reader: 18X DVD Rom - Writer: 24X DVD Recorder * 11 Disk - Writing Speed: DVD+R 24X / DVD-R 24X / CD-R 48X / DVD+RW 8X / DVD-RW 6X / CD-RW 32X Stand alone type | Set | 1 |  |  |
| 10 | CD/DVD Printer | - Supported Media: CDs, DVDs, Blu-ray Discs, and HD-DVDs - Disc Input: 100 - Printing Technology: Inkjet printer - Print Resolution: 4800 dpi x 1200dpi - Connectivity: Hi Speed USB Including Label Design S/W | Set | 1 |  |  |
| 11 | DAS /NAS Storage for RIC content backup | - 2.0GHz CPU, 512MB DDR2 Memory, 16MB Flash memory - HDD: 8TB(4 X 2TB, 3.5" SATA) - Tray: 4 X Hot Swappable Tray - Network: 2 Gigabit Ethernet - USB: 4 X USB 2.0 - Form Factor: Tower | Set | 1 |  |  |
| 12 | Software | MS-Office, Office University 2010 or above (latest) | Set | 10 |  |  |
|  |  | Creative Suite 6 Design & Web Premium Student and Teacher Edition or above (latest) | Set | 2 |  |  |
|  |  | Creative Suite 6 Production Premium Student and Teacher Edition or above (latest) | Set | 2 |  |  |
|  |  | Creative Suite 6 Master Collection Student and Teacher Edition or above (latest) | Set | 2 |  |  |
|  |  | Autodesk 3D Max Design 2013, Subscription (1 year) or above (latest) | Set | 1 |  |  |
|  |  | Corel painter 12 or above (latest) | Set | 2 |  |  |
|  |  | CorelDRAW Graphics Suite X6 or above (latest) | Set | 2 |  |  |
|  |  | Sound Forge Pro 10 or above (latest) | Set | 2 |  |  |
| 13 | Blank DVD | - DVD-R, 4.7GB 16X 50P, Printable | Set | 800 |  |  |
| 14 | Power & Network Cabling | - Network Cabling(minimum 14 Port): Cat 7A and miscellaneous materials - Power Cabling: HIV 600 heat-resistant Cable and power plugs of Colombian Type - Comply with electricity standards of Colombia | Set | 1 |  |  |

- Interior

| No | Equipment | Specification | Unit | Q'ty | Compliance (Y/N) Partially compliant | Bidder’s Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Furniture | - Office Desk: 10EA - Partition - Chair: 18EA - Printer Desk: 2EA - Office Cabinet: 8EA - Conference table(8person) 1EA - File Cabinet: 10EA * Reference only and minimum requirements | Set | 1 |  |  |
| 2 | Interior | - Wall, Roof, Window, etc - Air Conditioner *The Bidder shall propose a space plan. | Set | 1 |  |  |

# 6) Supporting Office (Central RIC)

- H/W and S/W

| No | Equipment | Specification | Unit | Q'ty | Compliance (Y/N) Partially compliant | Bidder’s Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Operator Computer | - Processor: Intel core i5-2400 processor (3.10 GHZ,6MB) and above - Graphics: 1GB or above - Memory: 6GB(DDR3 1333MHZ/4GB +2GB) - HDD: 1TB (7200RPM S-ATA2) - Monitor: 22" LCD Monitor * 2EA - O/S: Windows 7 Home Premium(64-bit) - Keyboard: Spanish and English - Mouse: USB Type Including Anti-Virus Program | Set | 19 |  |  |
| 2 | Notebook computer | - OS: Windows 8 (64bit) - CPU: Intel Core i5 Processor 3317U, up to 2.60GHz, 3MB cache or higher - Screen Size: 14" - Resolution: 1366 X 768 - Memory: 6GB DDR3 - HDD: 500GB(5400RPM, SATA) - ODD: DVD +/- RW - Speakers: Stereo Speaker - Web Camera: HD Camera - Connectivity: 802.11 b/g/n, Wired Ethernet LAN, Bluetooth - Keyboard: Spanish or English - Touch Pad Including AC Adapter, Wireless Optical USB Mouse, Anti-Virus S/W | Set | 8 |  |  |
| 3 | Potable Station | - Enough storage to accommodate 8 or more notebooks - Portable interface - Charging system - Security lock | Set | 1 |  |  |
| 4 | Software | MS-Office, Office University 2010 or above (latest) | Set | 27 |  |  |
|  |  | Creative Suite 6 Design & Web Premium Student and Teacher Edition or above (latest) | Set | 4 |  |  |
|  |  | Creative Suite 6 Production Premium Student and Teacher Edition or above (latest) | Set | 5 |  |  |
|  |  | Creative Suite 6 Master Collection Student and Teacher Edition or above (latest) | Set | 4 |  |  |
|  |  | Corel painter 12 or above (latest) | Set | 1 |  |  |
|  |  | CorelDRAW Graphics Suite X6 or above (latest) | Set | 1 |  |  |
| 5 | Power & Network Cabling | - Network Cabling(minimum 27 Port): Cat 7A and miscellaneous materials - Power Cabling: HIV 600 heat-resistant Cable and power plugs of Colombian Type - Comply with electricity standards of Colombia | Set | 1 |  |  |

- Interior

| No | Equipment | Specification | Unit | Q'ty | Compliance (Y/N) Partially compliant | Bidder’s Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Furniture | - Office Desk: 27EA - Chair: 27EA * Reference only and minimum requirements | Set | 1 |  |  |
| 2 | Interior | - Air Conditioner | Set | 1 |  |  |

# 7) Supporting Office (Other 4RIC)

- H/W and S/W

| No | Equipment | Specification | Unit | Q'ty | Compliance (Y/N) Partially compliant | Bidder’s Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Operator Computer | - Processor: Intel core i5-2400 processor (3.10 GHZ,6MB) and above - Graphics: 1GB or above - Memory: 6GB(DDR3 1333MHZ/4GB +2GB) - HDD: 1TB (7200RPM S-ATA2) - Monitor: 22" LCD Monitor * 2EA - O/S: Windows 7 Home Premium(64-bit) - Keyboard: Spanish and English - Mouse: USB Type Including Anti-Virus Program | Set | 14 |  |  |
| 2 | Notebook computer | - OS: Windows 8 (64bit) - CPU: Intel Core i5 Processor 3317U, up to 2.60GHz, 3MB cache or higher - Screen Size: 14" - Resolution: 1366 X 768 - Memory: 6GB DDR3 - HDD: 500GB(5400RPM, SATA) - ODD: DVD +/- RW - Speakers: Stereo Speaker - Web Camera: HD Camera - Connectivity: 802.11 b/g/n, Wired Ethernet LAN, Bluetooth - Keyboard: Spanish or English - Touch Pad Including AC Adapter, Wireless Optical USB Mouse, Anti-Virus S/W | Set | 8 |  |  |
| 3 | Potable Station | - Enough storage to accommodate 8 or more notebooks - Portable interface - Charging system - Security lock | Set | 1 |  |  |
| 4 | Software | MS-Office, Office University 2010 or above (latest) | Set | 22 |  |  |
|  |  | Creative Suite 6 Design & Web Premium Student and Teacher Edition or above (latest) | Set | 3 |  |  |
|  |  | Creative Suite 6 Production Premium Student and Teacher Edition or above (latest) | Set | 3 |  |  |
|  |  | Creative Suite 6 Master Collection Student and Teacher Edition or above (latest) | Set | 3 |  |  |
|  |  | Corel painter 12 or above (latest) | Set | 1 |  |  |
|  |  | CorelDRAW Graphics Suite X6 or above (latest) | Set | 1 |  |  |
| 5 | Power & Network Cabling | - Network Cabling(minimum 22 Port): Cat 7A and miscellaneous materials - Power Cabling: HIV 600 heat-resistant Cable and power plugs of Colombian Type - Comply with electricity standards of Colombia | Set | 1 |  |  |

- Interior

| No | Equipment | Specification | Unit | Q'ty | Compliance (Y/N) Partially compliant | Bidder’s Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Furniture | - Office Desk: 22EA - Chair: 22EA * Reference only and minimum requirements | Set | 1 |  |  |
| 2 | Interior | - Air Conditioner | Set | 1 |  |  |

# 8) Multimedia Classroom Type A

- H/W and S/W

| No | Equipment | Specification | Unit | Q'ty | Compliance (Y/N) Partially compliant | Bidder’s Comments |
| --- | --- | --- | --- | --- | --- | --- |
|  | A. Electronic Desk Board |  |  |  |  |  |
| 1 | PC for Trainer | - Processor: Intel core i5-2400 processor(3.10 GHZ,6MB) and above - Graphics: 1GB or above - Memory: 6GB(DDR3 1333MHZ/4GB +2GB) - HDD: 1TB (7200RPM S-ATA2) - Monitor: 22" LCD Monitor - O/S: Windows 7 Home Premium(64-bit) - Keyboard: Spanish and English - Mouse: USB Type Including Anti-Virus Program &HD Encoder | Set | 1 |  |  |
| 2 | PC for Trainee | - Processor: Intel core i5-2400 processor(3.10 GHZ,6MB) and above - Graphics: 1GB or above - Memory: 6GB(DDR3 1333MHZ/4GB +2GB) - HDD: 1TB (7200RPM S-ATA2) - Monitor: 22" LCD Monitor - O/S: Windows 7 Home Premium(64-bit) - Keyboard: Spanish and English - Mouse: USB Type Including Anti-Virus Program | Set | 30 |  |  |
| 3 | Multi Function Color Printer | Color Print, Copy, Scan, Fax Printing - Print Speed (Black): 21ppm - Print Speed (Color): 21ppm - Paper Input Capacity (Standard): 250 sheets Copy - Copy Speed (Black): 21cpm - Copy Speed (Color): 21cpm  Scan - Method: Color CCD or CIS or Flatbed - Scan Speed(Black): 11ipm or higher Fax - Modem Speed (kbps): 33.6 - Compatibility: ITU-T G3 - Fax Memory: 4MB or up to 250pages General - Interface: USB 2.0, Ethernet 10/100 Base TX - Memory: 256MB | Set | 2 |  |  |
| 4 | Software | MS-Office, Office University 2010 | Set | 31 |  |  |
| 5 | e-Podium | - Type: Stand Type - Top unit equipped with a minimum 21” writable LCD touch monitor. - PC and monitor locking system to prevent theft. - Built-in board cable and microphone Including Gooseneck Microphone, content authoring S/W * This product shall come from the same manufacturer of the Interactive Display Board. | Set | 1 |  |  |
| 6 | Interactive Display Board | - Size: 82", LED Panel - 16:9 aspect ratio - Resolution: 1920×1080(1080P, Full HD - Brightness: 450(cd/㎡) - Contrast : 5000:1 - PC Interface : USB, Plug and Play Including cables and accessories, S/W | Set | 1 |  |  |
|  | B. Audio System |  |  |  |  |  |
| 1 | Wireless Microphone | Handheld Transmitter - Audio Input Level: +2 dBV maximum at -10dB position - Gain Adjustment Range: 10dB - RF Transmitter Output: 30mW Maximum - Battery Life 8 hours (alkaline) - Including Microphone Receiver: - XLR connector (into 600 Ω load): –13 dBV - XLR Out: Impedance balanced Including external power supply *Comply with frequency rate of Colombia | Set | 2 |  |  |
| 2 | Wired Microphone | - Type: Dynamic - Frequency Response: 50 to 15,000 Hz - Polar Pattern: Cardioid - Sensitivity: –54.5 dBV/Pa (1.85 mV) Including 10m Cable | Set | 2 |  |  |
| 3 | DSP Mixer | - Acoustical Echo Cancellation - Noise Cancellation - Built-in telephone interface and 35W power amplication Audio Performance - Frequency Response: 20Hz to 22KHz ±1dB - Dynamic Range: 105dB(non A-weighted) Matrix Mixing Parameters - 12 analog in/out - 18 expansion bus in/out - 8 assignable processing blocks in/out - 4X35W speaker amplication - 1 telco input/1 telco out Control - 10/100 Mbps Ethernet Network Port and PC Port - RS-232(DB-9 female) - USB 2.0 compatible Including Configuration and Admin Software | Set | 1 |  |  |
| 4 | Ceiling Speaker | - Frequency range (-10 dB): 80 Hz - 20 kHz - Power handling (pink noise): 20 W - Sensitivity: 86 dB, 1 W @ 1 m, 1/2 space - Nominal impedance: 8 Ω | Set | 8 |  |  |
|  | C. Video System |  |  |  |  |  |
| 1 | HD Matrix Switcher | - RGBHV+Stereo, 8in 8out - RS-232 Control Audio - Input Level(max): +14dBu, unbalanced - Input Impedance: 18KΩ - Output Level(max): +14dBu, unbalanced - Output Impedance: 50Ω - Connectors: 3T Wideband Video - Input Level(max): +/- 1.5V - Input Impedance: 75Ω - Frequency Response: 300MHz or more (+/- 3dB) - HV Sync Input Level(max): +/- 5V - HV Sync Output Level(max): +/- 5V - Connectors: HD-15 | Set | 1 |  |  |
| 2 | Visual Presenter | - Capture area : 420 x 336(mm) - Focus : Auto / Manual - Camera Module: 42X Zoom, 1,300,000 pixels - Input: VGA(15pin D-Sub - Output: VGA(15pin D-Sub), DVI - Interface: RS-232C, USB, SD Memory | Set | 1 |  |  |
| 3 | Video Conference System | Video Standards and Protocols - H.264 / H.263++ / H.261 / H.239 / People+Content / H.263 & H.264 Video Error Concealment People Video Resolution - 720p, 30fps at 1 Mbps–2 Mbps - 4SIF (704 x 480)/4CIF (704 x 576) at 256 Kbps-2 Mbps - SIF (352 x 240), CIF (352 x 288) - QSIF (176 x 120), QCIF (176 x 144) Content Video Resolution - Up to 720p - Input: SXGA (1280x1024), XGA (1024 x 768),SVGA (800 x 600), VGA (640 x 480) - Output: XGA (1024x768), SVGA (800x600) Audio Standards and Protocols - Stereo Surround - 22 kHz bandwidth - 14 kHz bandwidth, G.722.1 Other ITU-Supported Standards - H.221 communications - H.224/H.281 far-end camera control - H.323 Annex Q far-end camera control - H.225, H.245, H.241, H.331 - H.239 dual stream - H.231 in multipoint calls - H.243 chair control - H.460 NAT/firewall traversal - BONDING, Mode 1 including document share function | Set | 1 |  |  |
| 4 | Camera | PTZ Camera - Image device: 1/3 type, CMOS - Video signal: 1080/59.94i, 1080/50i. 1080/29.97p, 1080/25p, 720/59.94p,720/50p/720/29.97p,720/25p ntsc/pal - Lens: 10x optical/ 4x digital - Video out: HD-SDI, Analog Component | Set | 2 |  |  |
| 5 | Motion Detection Camera System | Sensor Camera - CCD Fix Type Camera - 1/3” High Resolution CCD - Total Pixels: 811(H)x506(V) - Effective Pixels: 768(H)x494(V) - Resolution: More than 630TV Lines Motion Tracking Controller - E6600CPU, 1GB SDRAM, 120GB HDD, Composite Video Input, Rack Type - Motion tracking Control H/W - RS-232C, USB interface Including Cam Motion S/W | Set | 1 |  |  |
| 6 | Monitor | - Screen Size: 42" - Resolution: 1920 x 1080p - Display Type: LED - 4 HDMI port Including Ceiling Type Bracket | Set | 2 |  |  |
|  | D. Classroom Control |  |  |  |  |  |
| 1 | Controller | - 7 Configurable RS-232 / RS-422 / RS-485 Serial ports - 8 Relays - 8 IR / Serial ports - 8 Digital I/O ports - 2 Communication Networks: AxLink and Ethernet (TCP/IP) - 404 MIPS processor speed - 64 MB RAM or 256 MB RAM - 2 GB CompactFlash - 1 MB Non-Volatile Memory | Set | 1 |  |  |
| 2 | Lecture Touch Control Panel | - Compact 4.3" size - 16x9 WQVGA color display - Screen resolution (HV): 480 x 272 pixels - Available in a high-gloss and textured matte combination finish in your choice of black or white - Power over Ethernet (PoE) - 128 MB Standard memory / 256 MB Flash memory - 1 capacitive-touch external button | Set | 1 |  |  |
| 3 | Wireless Touch Panel | - Retina display - 9.7-inch (diagonal) LED-backlit glossy widescreen Multi-Touch display with IPS technology - 30-pin dock connector port - 3.5-mm stereo headphone minijack - Built-in speaker - Microphone | Set | 1 |  |  |
| 4 | iPad License | - Administrators shall be able to control and manage the equipment through iPad. | Set | 1 |  |  |
| 5 | Program Fee | - Audio/Visual equipment on/off control - Camera Pan/Tilt/Zoom control - Signal input/output control through HD Matrix Switcher and DSP Mixer | Set | 1 |  |  |
| 6 | Classroom Management S/W | - 1 Teacher and 30 Student License Recovery management function - System recovery/Existing status store - Reserved recovery set-up - Recovery install/delete PC management function - Screen lock/unlock - Internet lock/unlock - Registry lock/unlock - Task management lock/unlock - Program execution lock/unlock Instruction tool - Sending message - Sending notice message - Remote execution - Attendance check/Attendance data print - Client monitoring | Set | 1 |  |  |
|  | E. Miscellaneous |  |  |  |  |  |
| 1 | UPS | - Output Power Capacity: 10 KVA - Input - Voltage Range: 90~130AVC - Frequency: 50/60Hz ± 5% - Max Input Current: 112A - Output - Voltage Range: 110AVC± 1% - Frequency: 50/60Hz ± 5% - AVR Function: included - Battery Type Maintenance-free sealed Lead-Acid battery - Typical recharge time: 90% capacity after 8 hours - Surge Protection - Backup Time: 30 minutes or less - Comply with electricity standards of Colombia | Set | 1 |  |  |
| 2 | UPS for PC | - Output Capacity: 450Watts / 750 VA - Max Configurable Power: 450 Watts / 750 VA - Input /Output Voltage: 120V, 50/60Hz ± 3Hz - Battery Type: sealed Lead-Acid battery - Typical recharge time: 16hour - Backup Time at Half Load: 11minute(225 Watts) - Comply with electricity standards of Colombia | Set | 31 |  |  |
| 3 | Rack | - 19" 42U - Aluminum | Set | 1 |  |  |
| 4 | Power & Network Cabling | - Network Cabling(minimum 35ports): Cat 7A and miscellaneous materials - Power Cabling: HIV 600 heat-resistant Cable and power plugs of Colombian Type - Comply with electricity standards of Colombia | Set | 1 |  |  |

- Interior

| No | Equipment | Specification | Unit | Q'ty | Compliance (Y/N) Partially compliant | Bidder’s Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Furniture | - Office Desk for Trainees: 30EA - Chair: 31EA - Printer Desk: 2EA * Reference only and minimum requirements | Set | 1 |  |  |
| 2 | Interior | - Wall, Roof, Window, and etc - Air Conditioner * The Bidder shall propose a space plan. | Set | 1 |  |  |

# 9) Multimedia Classroom Type B

- H/W and S/W

| No | Equipment | Specification | Unit | Q'ty | Compliance (Y/N) Partially compliant | Bidder’s Comments |
| --- | --- | --- | --- | --- | --- | --- |
|  | A. Electronic Desk Board |  |  |  |  |  |
| 1 | PC for Trainer | - Processor: Intel core i5-2400 processor(3.10 GHZ,6MB) and above - Graphics: 1GB or above - Memory: 6GB(DDR3 1333MHZ/4GB +2GB) - HDD: 1TB (7200RPM S-ATA2) - Monitor: 22" LCD Monitor - O/S: Windows 7 Home Premium(64-bit) - Keyboard: Spanish and English - Mouse: USB Type Including Anti-Virus Program &HD Encoder | Set | 1 |  |  |
| 2 | PC for Trainee | - Processor: Intel core i5-2400 processor(3.10 GHZ,6MB) and above - Graphics: 1GB or above - Memory: 6GB(DDR3 1333MHZ/4GB +2GB) - HDD: 1TB (7200RPM S-ATA2) - Monitor: 22" LCD Monitor - O/S: Windows 7 Home Premium(64-bit) - Keyboard: Spanish and English - Mouse: USB Type Including Anti-Virus Program | Set | 30 |  |  |
| 3 | Multi Function Color Printer | Color Print, Copy, Scan, Fax Printing - Print Speed (Black): 21ppm - Print Speed (Color): 21ppm - Paper Input Capacity (Standard): 250 sheets Copy - Copy Speed (Black): 21cpm - Copy Speed (Color): 21cpm  Scan - Method: Color CCD or CIS or Flatbed - Scan Speed(Black): 11ipm or higher Fax - Modem Speed (kbps): 33.6 - Compatibility: ITU-T G3 - Fax Memory: 4MB or up to 250pages General - Interface: USB 2.0, Ethernet 10/100 Base TX - Memory: 256MB | Set | 2 |  |  |
| 4 | Software | MS-Office, Office University 2010 | Set | 31 |  |  |
| 5 | e-Podium | - Type: Stand Type - Top unit equipped with a minimum 21” writable LCD touch monitor. - PC and monitor locking system to prevent theft. - Built-in board cable and microphone Including Gooseneck Microphone, content authoring S/W * This product shall come from the same manufacturer of the Interactive Display Board. | Set | 1 |  |  |
| 6 | Interactive Display Board | - Size: 82", LED Panel - 16:9 aspect ratio - Resolution: 1920×1080(1080P, Full HD - Brightness: 450(cd/㎡) - Contrast : 5000:1 - PC Interface : USB, Plug and Play Including cables and accessories, S/W | Set | 1 |  |  |
|  | B. Audio System |  |  |  |  |  |
| 1 | Wireless Microphone | Handheld Transmitter - Audio Input Level: +2 dBV maximum at -10dB position - Gain Adjustment Range: 10dB - RF Transmitter Output: 30mW Maximum - Battery Life 8 hours (alkaline) - Including Microphone Receiver: - XLR connector (into 600 Ω load): –13 dBV - XLR Out: Impedance balanced Including external power supply *Comply with frequency rate of Colombia | Set | 2 |  |  |
| 2 | Wired Microphone | - Type: Dynamic - Frequency Response: 50 to 15,000 Hz - Polar Pattern: Cardioid - Sensitivity: –54.5 dBV/Pa (1.85 mV) Including 10m Cable | Set | 2 |  |  |
| 3 | DSP Mixer | - Acoustical Echo Cancellation - Noise Cancellation - Built-in telephone interface and 35W power amplication Audio Performance - Frequency Response: 20Hz to 22KHz ±1dB - Dynamic Range: 105dB(non A-weighted) Matrix Mixing Parameters - 12 analog in/out - 18 expansion bus in/out - 8 assignable processing blocks in/out - 4X35W speaker amplication - 1 telco input/1 telco out Control - 10/100 Mbps Ethernet Network Port and PC Port - RS-232(DB-9 female) - USB 2.0 compatible Including Configuration and Admin Software | Set | 1 |  |  |
| 4 | Ceiling Speaker | - Frequency range (-10 dB): 80 Hz - 20 kHz - Power handling (pink noise): 20 W - Sensitivity: 86 dB, 1 W @ 1 m, 1/2 space - Nominal impedance: 8 Ω | Set | 8 |  |  |
|  | C. Video System |  |  |  |  |  |
| 1 | HD Matrix Switcher | - RGBHV+Stereo, 8in 8out - RS-232 Control Audio - Input Level(max): +14dBu, unbalanced - Input Impedance: 18KΩ - Output Level(max): +14dBu, unbalanced - Output Impedance: 50Ω - Connectors: 3T Wideband Video - Input Level(max): +/- 1.5V - Input Impedance: 75Ω - Frequency Response: 300MHz or more (+/- 3dB) - HV Sync Input Level(max): +/- 5V - HV Sync Output Level(max): +/- 5V - Connectors: HD-15 | Set | 1 |  |  |
| 2 | Visual Presenter | - Capture area : 420 x 336(mm) - Focus : Auto / Manual - Camera Module: 42X Zoom, 1,300,000 pixels - Input: VGA(15pin D-Sub - Output: VGA(15pin D-Sub), DVI - Interface: RS-232C, USB, SD Memory | Set | 1 |  |  |
|  | D. Classroom Control |  |  |  |  |  |
| 1 | Controller | - 7 Configurable RS-232 / RS-422 / RS-485 Serial ports - 8 Relays - 8 IR / Serial ports - 8 Digital I/O ports - 2 Communication Networks: AxLink and Ethernet (TCP/IP) - 404 MIPS processor speed - 64 MB RAM or 256 MB RAM - 2 GB CompactFlash - 1 MB Non-Volatile Memory | Set | 1 |  |  |
| 2 | Lecture Touch Control Panel | - Compact 4.3" size - 16x9 WQVGA color display - Screen resolution (HV): 480 x 272 pixels - Available in a high-gloss and textured matte combination finish in your choice of black or white - Power over Ethernet (PoE) - 128 MB Standard memory / 256 MB Flash memory - 1 capacitive-touch external button | Set | 1 |  |  |
| 3 | Wireless Touch Panel | - Retina display - 9.7-inch (diagonal) LED-backlit glossy widescreen Multi-Touch display with IPS technology - 30-pin dock connector port - 3.5-mm stereo headphone minijack - Built-in speaker - Microphone | Set | 1 |  |  |
| 4 | iPad License | - Administrators shall be able to control and manage the equipment through iPad. | Set | 1 |  |  |
| 5 | Program Fee | - Audio/Visual equipment on/off control - Camera Pan/Tilt/Zoom control - Signal input/output control through HD Matrix Switcher and DSP Mixer | Set | 1 |  |  |
| 6 | Classroom Management S/W | - 1 Teacher and 30 Student License Recovery management function - System recovery/Existing status store - Reserved recovery set-up - Recovery install/delete PC management function - Screen lock/unlock - Internet lock/unlock - Registry lock/unlock - Task management lock/unlock - Program execution lock/unlock Instruction tool - Sending message - Sending notice message - Remote execution - Attendance check/Attendance data print - Client monitoring | Set | 1 |  |  |
|  | E. Miscellaneous |  |  |  |  |  |
| 1 | UPS | - Output Power Capacity: 10 KVA - Input - Voltage Range: 90~130AVC - Frequency: 50/60Hz ± 5% - Max Input Current: 112A - Output - Voltage Range: 110AVC± 1% - Frequency: 50/60Hz ± 5% - AVR Function: included - Battery Type Maintenance-free sealed Lead-Acid battery - Typical recharge time: 90% capacity after 8 hours - Surge Protection - Backup Time: 30 minutes or less - Comply with electricity standards of Colombia | Set | 1 |  |  |
| 2 | UPS for PC | - Output Capacity: 450Watts / 750 VA - Max Configurable Power: 450 Watts / 750 VA - Input /Output Voltage: 120V, 50/60Hz ± 3Hz - Battery Type: sealed Lead-Acid battery - Typical recharge time: 16hour - Backup Time at Half Load: 11minute(225 Watts) - Comply with electricity standards of Colombia | Set | 31 |  |  |
| 3 | Rack | - 19" 42U - Aluminum | Set | 1 |  |  |
| 4 | Power & Network Cabling | - Network Cabling(minimum 35ports): Cat 7A and miscellaneous materials - Power Cabling: HIV 600 heat-resistant Cable and power plugs of Colombian Type - Comply with electricity standards of Colombia | Set | 1 |  |  |

- Interior

| No | Equipment | Specification | Unit | Q'ty | Compliance (Y/N) Partially compliant | Bidder’s Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Furniture | - Office Desk for Trainees: 30EA - Chair: 31EA - Printer Desk: 2EA * Reference only and minimum requirements | Set | 1 |  |  |
| 2 | Interior | - Wall, Roof, Window, and etc - Air Conditioner * The Bidder shall propose a space plan. | Set | 1 |  |  |

# 10) General Classroom

- H/W and S/W

| No | Equipment | Specification | Unit | Q'ty | Compliance (Y/N) Partially compliant | Bidder’s Comments |
| --- | --- | --- | --- | --- | --- | --- |
|  | A. Electronic Desk Board |  |  |  |  |  |
| 1 | PC for Trainer | - Processor: Intel core i5-2400 processor(3.10 GHZ,6MB) and above - Graphics: 1GB and above - Memory: 6GB(DDR3 1333MHZ/4GB +2GB) - HDD: 1TB (7200RPM S-ATA2) - Monitor: 22" LCD Monitor - O/S: Windows 7 Home Premium(64-bit) - Keyboard: Spanish and English - Mouse: USB Type Including Anti-Virus Program | Set | 1 |  |  |
| 2 | PC for Trainee | - Processor: Intel core i5-2400 processor(3.10 GHZ,6MB) and above - Graphics: 1GB and above - Memory: 6GB(DDR3 1333MHZ/4GB +2GB) - HDD: 1TB (7200RPM S-ATA2) - Monitor: 22" LCD Monitor - O/S: Windows 7 Home Premium(64-bit) - Keyboard: Spanish and English - Mouse: USB Type - Including Anti-Virus Program | Set | 15 |  |  |
| 3 | Multi Function Color Printer | Color Print, Copy, Scan, Fax Printing - Print Speed (Black): 21ppm - Print Speed (Color): 21ppm - Paper Input Capacity (Standard): 250 sheets Copy - Copy Speed (Black): 21cpm - Copy Speed (Color): 21cpm  Scan - Method: Color CCD or CIS or Flatbed - Scan Speed(Black): 11ipm or higher Fax - Modem Speed (kbps): 33.6 - Compatibility: ITU-T G3 - Fax Memory: 4MB or up to 250pages General - Interface: USB 2.0, Ethernet 10/100 Base TX - Memory: 256MB | Set | 2 |  |  |
| 4 | Software | MS-Office, Office University 2010 | Set | 16 |  |  |
| 5 | e-Podium | - Type: Stand Type - Top unit equipped with a minimum 21” writable LCD Tablet monitor. - PC and monitor locking system to prevent theft. - Built-in board cable and microphone Including Gooseneck Microphone, content authoring S/W - This product shall come from the same manufacturer of the Interactive Display Board. | Set | 1 |  |  |
| 6 | Video Projector and Screen | Projector - Native Resolution: XGA (1024x768) - Brightness (ANSI-Lumens): 4000ANSI - Contrast Ratio (FIFO): 2000:1 and above - Lamp life (hours): 2500 and above Screen - Size: 120”(2400 x 1800) - Material: Matt White - Motorized | Set | 1 |  |  |
|  | B. Audio System |  |  |  |  |  |
| 1 | Wireless Microphone | Handheld Transmitter - Audio Input Level: +2 dBV maximum at -10dB position - Gain Adjustment Range: 10dB - RF Transmitter Output: 30mW Maximum - Battery Life 8 hours (alkaline) - Including Microphone Receiver: - XLR connector (into 600 Ω load): –13 dBV - XLR Out: Impedance balanced Including external power supply Comply with frequency rate of Colombia | Set | 2 |  |  |
| 2 | Wired Microphone | - Type: Dynamic - Frequency Response: 50 to 15,000 Hz - Polar Pattern: Cardioid - Sensitivity: –54.5 dBV/Pa (1.85 mV) | Set | 2 |  |  |
| 3 | DSP Mixer | - Acoustical Echo Cancellation - Noise Cancellation - Built-in telephone interface and 35W power amplication Audio Performance - Frequency Response: 20Hz to 22KHz ±1dB - Dynamic Range: 105dB(non A-weighted) Matrix Mixing Parameters - 12 analog in/out - 18 expansion bus in/out - 8 assignable processing blocks in/out - 4X35W speaker amplication - 1 telco input/1 telco out Control - 10/100 Mbps Ethernet Network Port and PC Port - RS-232(DB-9 female) - USB 2.0 compatible Including Configuration and Admin Software | Set | 1 |  |  |
| 4 | Ceiling Speaker | - Frequency range (-10 dB): 80 Hz - 20 kHz - Power handling (pink noise): 20 W - Sensitivity: 86 dB, 1 W @ 1 m, 1/2 space - Nominal impedance: 8 Ω | Set | 8 |  |  |
|  | C. Video System |  |  |  |  |  |
| 1 | HD Matrix Switcher | - RGBHV+Stereo, 8in 8out - RS-232 Control Audio - Input Level(max): +14dBu, unbalanced - Input Impedance: 18KΩ - Output Level(max): +14dBu, unbalanced - Output Impedance: 50Ω - Connectors: 3T Wideband Video - Input Level(max): +/- 1.5V - Input Impedance: 75Ω - Frequency Response: 300MHz or better(+/- 3dB) - HV Sync Input Level(max): +/- 5V - HV Sync Output Level(max): +/- 5V - Connectors: HD-15 | Set | 1 |  |  |
| 2 | Visual Presenter | - Capture area : 420 x 336(mm) - Focus : Auto / Manual - Camera Module: 42X Zoom, 1,300,000 pixels - Input: VGA(15pin D-Sub - Output: VGA(15pin D-Sub), DVI - Interface: RS-232C, USB, SD Memory | Set | 1 |  |  |
|  | D. Classroom Control |  |  |  |  |  |
| 1 | Controller | - 7 Configurable RS-232 / RS-422 / RS-485 Serial ports - 8 Relays - 8 IR / Serial ports - 8 Digital I/O ports - 2 Communication Networks: AxLink and Ethernet (TCP/IP) - 404 MIPS processor speed - 64 MB RAM or 256 MB RAM - 2 GB CompactFlash - 1 MB Non-Volatile Memory | Set | 1 |  |  |
| 2 | Lecture Touch Control Panel | - Compact 4.3" size - 16x9 WQVGA color display - Screen resolution (HV): 480 x 272 pixels - Available in a high-gloss and textured matte combination finish in your choice of black or white - Power over Ethernet (PoE) - 128 MB Standard memory / 256 MB Flash memory - 1 capacitive-touch external button | Set | 1 |  |  |
| 3 | Wireless Touch Panel | - Retina display - 9.7-inch (diagonal) LED-backlit glossy widescreen Multi-Touch display with IPS technology - 30-pin dock connector port - 3.5-mm stereo headphone mini jack - Built-in speaker - Microphone | Set | 1 |  |  |
| 4 | iPad License | - Administrators shall be able to control and manage the equipment through iPad. | Set | 1 |  |  |
| 5 | Program Fee | - Audio/Visual equipment on/off control - Camera Pan/Tilt/Zoom control - Signal input/output control through HD Matrix Switcher and DSP Mixer | Set | 1 |  |  |
| 6 | Classroom Management S/W | - 1 Teacher and 15 Student License Recovery management function - System recovery/Existing status store - Reserved recovery set-up - Recovery install/delete PC management function - Screen lock/unlock - Internet lock/unlock - Registry lock/unlock - Task management lock/unlock - Program execution lock/unlock Instruction tool - Sending message - Sending notice message - Remote execution - Attendance check/Attendance data print - Client monitoring | Set | 1 |  |  |
|  | E. Miscellaneous |  |  |  |  |  |
| 1 | UPS | - Output Power Capacity: 10 KVA - Input - Voltage Range: 90~130AVC - Frequency: 50/60Hz ± 5% - Max Input Current: 112A - Output - Voltage Range: 110AVC± 1% - Frequency: 50/60Hz ± 5% - AVR Function: included - Battery Type Maintenance-free sealed Lead-Acid battery - Typical recharge time: 90% capacity after 8 hours - Surge Protection - Backup Time: 30 minutes or less - Comply with electricity standards of Colombia | Set | 1 |  |  |
| 2 | UPS for PC | - Output Capacity: 450Watts / 750 VA - Max Configurable Power: 450 Watts / 750 VA - Input /Output Voltage: 120V, 50/60Hz ± 3Hz - Battery Type: sealed Lead-Acid battery - Typical recharge time: 16hour - Backup Time at Half Load: 11minute(225 Watts) - Comply with electricity standards of Colombia | Set | 16 |  |  |
| 3 | Rack | - 19" 42U - Aluminum | Set | 1 |  |  |
| 4 | Power & Network Cabling | - Network Cabling(minimum 19 Ports): Cat 7A and miscellaneous materials - Power Cabling: HIV 600 heat-resistant Cable and power plugs of Colombian Type - Comply with electricity standards of Colombia | Set | 1 |  |  |

- Interior

| No | Equipment | Specification | Unit | Q'ty | Compliance (Y/N) Partially compliant | Bidder’s Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Furniture | - Office Desk for Trainee: 15EA - Chair: 31EA - Printer Desk: 2EA * Reference only and minimum requirements | Set | 1 |  |  |
| 2 | Interior | - Wall, Roof, Window, etc - Air Conditioner * The Bidder shall propose a space plan. | Set | 1 |  |  |

# 11) RIC Security System

- H/W and S/W

| No | Equipment | Specification | Unit | Q'ty | Compliance (Y/N) Partially compliant | Bidder’s Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Camera | - Image Sensor: 1/3”CCD - Pixels: 410,000 Pixels, 811(H) X 494(V) - Horizontal Resolution: 600TV Line(Color), 700TV Line(B/W) - Video Out: CVBS: 1.0p_p, Composite - S/N Ration: 52dB - Minimum Illumination(B/W): 0.01Lux(50IRE@F1.2), 0.00002Lux(Sens-up, x512) - OSD: Spanish | Set | 10 |  |  |
| 2 | Motion Detection Sensor | Motion Detection Sensor (8 Set) - Sensing Method: Infrared - Detection Range: 12m X 1m - Mount Type: Ceiling Type - Alarm: NC/NO Door Sensor(10Set) - Sensing Distance: 25~55mm - Installation Location: In the Door or Window - Materials: Cover(ABS), Magnet * Required number and kind of equipment may vary depending on the actual space of RIC. | Set | 1 |  |  |
| 3 | DVR | - Video Input: Composite 16CH - Monitor Output - Main Composite; BNC, 1 Vp-p, 75 ohm - VGA: 800X600, 1024X768, 1280X1024 - HDMI: 480P/720P/1080P(1920X1080) - Audio: 16CH In/1CH Out - Recoding - Compression: H.264 - Resolution: up to 480FPS @ 704X480 - Searching: Date/time, Event, Back up, Pos, Motion (*All Search Included Preview Function) - Alarm in/out: 16 line in / 4 relay out - HDD: 1TB - Network - Network: 10/100/1000Mbps - USB 2.0: 3 Ports - Web Viewer Function - PTZ Control:RS-485 - Serial Interface: RS-232 including 17” Monitor | Set | 1 |  |  |
| 4 | UPS | - Output Power Capacity: 5 KVA - Input - Voltage Range: 100 ~ 280V - Frequency: 50/60Hz ± 5% - Output - Voltage Range: 120V - Frequency: 50/60Hz ± 3Hz - Battery Type Maintenance-free sealed Lead-Acid battery - Typical recharge time: 2.5 hours - Surge Protection and Filtering: Included - Backup Time: 18 minutes(at 1,750Watts) or more - Comply with electricity standards of Colombia | Set | 1 |  |  |
| 5 | Rack | - 19", 22U - Aluminum | Set | 1 |  |  |

# 3. Lab School

-H/W and S/W

| No | Equipment | Specification | Unit | Q'ty | Compliance (Y/N) Partially compliant | Bidder’s Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | F/W | - Anti-Virus - Child Protection - 1Year License for Server | Set | 50 |  |  |
| 2 | Network Switch | - Ports: 22 autosensing 10/100/1000 ports or more - Mounting: EIA-standard 19" Rack - Memory and processor: 8 MB flash, packet buffer size: 512 KB, 16 MB SDRAM - Throughput: up to 35.7 million pps (64-byte packets) - Management features: Web browser Switching capacity: 48 Gbps | Set | 50 |  |  |
| 3 | Proxy Server | - OS: WIndows7 Professional 64 - Processor: Intel Xeon Quad-Core W3520(2.66GHz, 8MB cache, 1066Mhz Memory) * 1EA - Memory: 4GB DDR3 - HDD: 500GB 7200rpm SATA - Optical Drive: SATA SuperMulti-DVD Writer - Graphic Memory: 512MB - Network: 10/100/1000Mbps - Keyboard and Mouse including 22" LCD Monitor | Set | 50 |  |  |
| 4 | Access Point (WiFi) | - 802.11a/b/g, 54Mbps - Dual Antenna - Authentication: Security Standards, WPA, WPA2 (802.11i) - Encryption: AES encryption (WPA2), TKIP (WPA), WPA TKIP | Set | 300 |  |  |
| 5 | Notebook | - OS: Windows7 Home Premium (64bit) - CPU: Intel Core i5 Processor 3210M, 2.60Ghz, 3MB or higher - Screen Size: 15.6" - Resolution: 1366 X 768 - Memory: 6GB DDR3 - HDD: 500GB - ODD: DVD +/- RW - Speakers: Stereo Speaker - Web Camera: HD Camera - Connectivity: 802.11 b/g/n, Gigabit Wired Ethernet LAN, Bluetooth - Keyboard: 101 Key(Spanish) - Touch Pad Including AC Adapter, Optical USB Mouse, Anti-Virus S/W | Set | 1100 |  |  |
| 6 | Software | - MS-Office , Home and Student | Set | 1100 |  |  |
| 7 | Multi Function Printer | - Color Print, Copy, Scan Printing - Print Speed (Black): 17ppm or higher - Print Speed (Color): 4ppm Copy - Copy Speed (Black): 17cpm or higher - Copy Speed (Color): 4ppm Scan - Method: Color CIS or CCD or Flatbed - Resolution (Optical): up to 1200dpi or higher - Scan Speed: up to 14ppm General - Interface: High-Speed USB 2.0 / 10/100 Base Tx / Wireless 802.11 b/g/n - Memory: 128MB | Set | 100 |  |  |
| 8 | Toner | - Toner for Multifunction Color Printer(Black, Minimum Printing Capacity : 2,000pages): 300EA - Toner for Multifunction Color Printer(Yellow, Minimum Printing Capacity : 2,000pages): 300EA - Toner for Multifunction Color Printer(Magenta, Minimum Printing Capacity : 2,000pages): 300EA - Toner for Multifunction Color Printer(Cyan, Minimum Printing Capacity : 2,000pages): 300EA | Set | 1 |  |  |
| 9 | Projector and Portable Screen | Projector - Native Resolution: XGA(1024x768) - Brightness(ANSI-Lumens): 3200ANSI - Contrast Ratio(FIFO): 2000:1 and above - Lamp life(hours): 3000 and above Portable Screen - Size: 120”(2400 x 1800) - Material: Matt White | Set | 100 |  |  |
| 10 | Portable Station | - Enough storage to accommodate 11 or more notebooks - Portable interface - Charging system - Security lock | Set | 100 |  |  |
| 11 | UPS | - Capacity: 3000VA - Output Voltage: 120V - Output Frequency: 50 or 60Hz +/-10% - Voltage Regulation( AVR) function - Input Voltage: 120V - Input Frequency: 50 or 60Hz +/-3% - Battery type: Sealed, maintenance-free lead acid batteries with 3-6 years typical lifetime - Back-up Time(a PC with 15" monitor): 95~105 mins * Comply with electricity standards of Colombia | Set | 50 |  |  |
| 12 | Power Extension | - 6 Socket - 3M * Comply with electricity standards of Colombia | Set | 200 |  |  |
| 13 | Classroom Management S/W | - 2 Teacher License, 20 Student License Recovery management function - System recovery/Existing status store - Reserved recovery set-up - Recovery install/delete PC management function - Screen lock/unlock - Internet lock/unlock - Registry lock/unlock - Task management lock/unlock - Program execution lock/unlock Instruction tool - Sending message - Sending notice message - Remote execution - Attendance check/Attendance data print - Client monitoring | Set | 50 |  |  |